# ASSIGNMENT – 5

# Performing FMEA and FTA Exercises

### Case Study: Risk Analysis in Amul Dairy Production Expansion

# 1. Introduction

Risk analysis is an important aspect of production management, particularly in industries dealing with perishable goods. Dairy manufacturing requires continuous processing, temperature control, and efficient logistics. Any disruption in production, storage, or transportation can result in product spoilage and financial losses.

Two widely used techniques for analyzing industrial risks are Failure Mode and Effects Analysis (FMEA) and Fault Tree Analysis (FTA).

Failure Mode and Effects Analysis helps identify potential failures in a system and evaluates their severity, likelihood of occurrence, and ability to detect the failure before it causes damage. Fault Tree Analysis is a graphical approach that identifies the root causes of system failures by analyzing logical relationships between events.

This assignment applies FMEA and FTA techniques to the Amul dairy production system, using the risks previously identified in Assignment 4 during the analysis of production expansion.

# 2. Company Overview

Amul (Gujarat Cooperative Milk Marketing Federation – GCMMF) is one of India's largest dairy cooperatives headquartered in Anand, Gujarat. Established in 1946, the organization operates through a cooperative model connecting millions of dairy farmers to processing plants and national markets.

Key Highlights:

- Established: 1946

- Industry: Dairy and FMCG

- Farmer Network: 36+ lakh farmers

- Daily Milk Procurement: Approximately 23–29 million liters

- Major Products: Milk, butter, cheese, ice cream, ghee, milk powder

- Distribution: Nationwide retail network and export markets

Due to the highly perishable nature of dairy products, Amul's operations heavily depend on cold storage systems, efficient logistics, and continuous production processes. Any disruption in these systems can lead to product loss and operational instability.

# 3. Production Expansion Scenario

Between FY 2020 and FY 2023, Amul significantly increased its milk procurement capacity to meet rising demand for dairy products across India.

### Milk Procurement Growth

| Financial Year | Milk Procurement (Million Liters per Day) |
| --- | --- |
| FY 2020 | ~23 |
| FY 2021 | ~24 |
| FY 2022 | ~26 |
| FY 2023 | ~29 |

This increase in production required:

- Expansion of dairy processing facilities

- Installation of additional pasteurization equipment

- Expansion of cold storage infrastructure

- Increased transportation logistics

- Higher energy consumption

While expansion increases revenue potential, it also increases operational complexity and risk exposure.

# 4. Overview of Failure Mode and Effects Analysis (FMEA)

Failure Mode and Effects Analysis (FMEA) is a structured method used to identify possible failure modes within a system and evaluate their potential impact.

The method evaluates three parameters:

| Parameter | Description |
| --- | --- |
| Severity (S) | Impact of failure on operations |
| Occurrence (O) | Probability of failure happening |
| Detection (D) | Ability to detect failure before impact |

The Risk Priority Number (RPN) is calculated as:

RPN = Severity × Occurrence × Detection

Higher RPN values indicate higher risk priority.

# 5. FMEA Rating Scales

## Severity Scale

| Rating | Description | Effect |
| --- | --- | --- |
| 1–2 | Very Low | No operational impact |
| 3–4 | Low | Minor production disturbance |
| 5–6 | Moderate | Production slowdown |
| 7–8 | High | Significant production loss |
| 9–10 | Very High | Production shutdown or major financial loss |

## Occurrence Scale

| Rating | Description | Frequency |
| --- | --- | --- |
| 1–2 | Rare | Failure unlikely |
| 3–4 | Low | Occasional failure |
| 5–6 | Moderate | Occurs sometimes |
| 7–8 | High | Frequent failure |
| 9–10 | Very High | Almost certain |

## Detection Scale

| Rating | Description | Detection Capability |
| --- | --- | --- |
| 1–2 | Very High | Easily detected |
| 3–4 | High | Early detection possible |
| 5–6 | Moderate | May be detected |
| 7–8 | Low | Hard to detect |
| 9–10 | Very Low | Almost impossible to detect |

# 6. FMEA Analysis Based on Risk Register (10 Risks)

| Failure Mode | Possible Cause | Effect on System | Severity | Occurrence | Detection | RPN | Recommended Action |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Processing overload / machine breakdown (R1) | Continuous high production load | Production halt and milk spoilage | 9 | 6 | 4 | 216 | Preventive maintenance |
| Cold storage compressor failure (R2) | Equipment malfunction | Milk spoilage | 10 | 5 | 4 | 200 | Backup refrigeration |
| Milk procurement imbalance (R3) | Seasonal variation | Production imbalance | 7 | 6 | 5 | 210 | Diversify farmer network |
| Power outage during peak load (R4) | Electrical grid failure | Refrigeration shutdown | 9 | 4 | 5 | 180 | Backup generators |
| Quality inspection delay (R5) | Increased production speed | Contamination risk | 10 | 3 | 6 | 180 | Automated testing |
| Logistics disruption (R6) | Transport delays | Delivery delays | 6 | 6 | 6 | 216 | Route optimization |
| Fuel price increase (R7) | Market fluctuations | Higher transport cost | 5 | 5 | 6 | 150 | Long-term fuel contracts |
| Demand overestimation (R8) | Incorrect forecasting | Excess inventory | 6 | 4 | 6 | 144 | Market demand analysis |
| Packaging material shortage (R9) | Supplier disruption | Production slowdown | 5 | 4 | 6 | 120 | Multiple suppliers |
| Capital recovery delay (R10) | High infrastructure investment | Financial pressure | 6 | 3 | 7 | 126 | Cost control planning |

# 7. Interpretation of FMEA Results

Based on the Risk Priority Numbers, the most critical failure modes include:

- Processing equipment overload

- Logistics disruptions

- Milk procurement imbalance

- Cold storage system failure

These risks have the highest RPN values and therefore require priority attention in Amul's risk management strategy.

# 8. Introduction to Fault Tree Analysis (FTA)

Fault Tree Analysis (FTA) is a top-down method used to analyze the causes of system failures. The analysis begins with a top event, which represents the primary system failure, and then identifies the events that could cause it.

Logical relationships such as AND gates and OR gates are used to connect causes to the top event.

FTA helps visualize the relationship between system failures and their root causes.

# 9. Fault Tree Analysis for Amul Production System

### Top Event

Milk Spoilage in Processing Plant

Milk spoilage represents the most critical operational failure because dairy products have limited shelf life.

The top event may occur due to multiple contributing causes.

### Primary Causes

| Cause Category | Related Risk |
| --- | --- |
| Cold storage system failure | R2 |
| Processing equipment breakdown | R1 |
| Power outage | R4 |
| Logistics disruption | R6 |
| Quality inspection failure | R5 |

### Secondary Causes

| Failure Source | Description |
| --- | --- |
| Compressor malfunction | Cold storage failure |
| Equipment overload | Processing breakdown |
| Vehicle breakdown | Transport delay |
| Grid failure | Power outage |
| Inspection error | Quality failure |

# 10. Fault Tree Diagram

# 11. Fault Tree Interpretation

The Fault Tree Analysis identifies milk spoilage as the top-level system failure. The analysis shows that multiple operational and infrastructure failures can contribute to this event.

Among the causes, refrigeration failure, processing equipment malfunction, and logistics delays represent the most significant risks. These failures directly impact the safety and quality of dairy products.

FTA helps managers visualize these relationships and implement preventive actions such as backup systems, maintenance planning, and improved logistics coordination.

# 12. Failure Impact Comparison

| Failure Event | Production Impact | Financial Impact | Risk Priority |
| --- | --- | --- | --- |
| Cold storage failure | Very High | Very High | Critical |
| Processing equipment failure | High | High | Critical |
| Logistics disruption | Moderate | Medium | Medium |
| Packaging shortage | Low | Low | Low |

# 13. Risk Mitigation Strategies

To reduce operational risks, Amul can implement the following measures:

### Operational Controls

- Preventive maintenance for machinery

- Automation in processing systems

### Infrastructure Controls

- Backup power generators

- Redundant refrigeration systems

### Supply Chain Improvements

- GPS monitoring of transport vehicles

- Multiple logistics partners

### Quality Assurance

- Automated milk testing systems

- Strict hygiene and inspection procedures

# 14. Advantages of Using FMEA and FTA in Dairy Manufacturing

Applying FMEA and FTA techniques in dairy production systems provides several benefits, especially in industries dealing with perishable goods.

Key Advantages:

- Early Risk Identification: Helps identify potential failures in processing, storage, and logistics before they occur.

- Improved Product Safety: Ensures better quality control and reduces contamination risks.

- Reduced Financial Loss: Prevents large-scale spoilage of milk and dairy products.

- Better Maintenance Planning: Helps prioritize preventive maintenance for critical equipment such as pasteurizers and refrigeration systems.

- Enhanced Decision Making: Provides structured data to help management focus on high-priority risks.

- Operational Efficiency: Improves coordination between production, storage, and transportation systems.

By applying these techniques, dairy companies like Amul can improve operational reliability and maintain consistent product quality even during large-scale production expansion.

# 15. Conclusion

Production expansion increases operational complexity and introduces multiple potential failure points within manufacturing systems. The application of FMEA and FTA techniques allows organizations to systematically identify critical risks and understand their root causes.

The analysis of Amul's dairy production system indicates that failures in refrigeration systems, processing equipment, and logistics operations represent the most significant threats to production stability.

By implementing preventive maintenance, infrastructure redundancy, and improved supply chain management, Amul can effectively reduce operational risks and support sustainable production growth.